- Init calib complete has to be asserted before any command can be driven out.

- Registering the init_calib_complete to meet timing

- The app_en signal is used to qualify the command on the command bus to the MC.

- The command on the command bus is considered accepted by the MC when app_rdy signal from the UI is asserted.

- The app_en signal is asserted at the same clock cycle.

- The app_cmd signal is the command issued to the MC.

- The cmd is set to Write command till the required write commands are issued to the UI after calibration complete.

- The cmd is set to Read command after the required write commands are completed and that many read commands are issued to the UI.

- Only COLUMN address bits are incremented in example design

- The app_addr signal is the command address issued to the MC.

- The cmd_addr is initialised to BEGIN_ADDRESS and increments by 8(`h8) when command enable is '1' and MC command fifo is ready.

if(rst)

cmd_addr <= BEGIN_ADDRESS       where begin_address <= (others => ‘0’)

- The cmd_addr is initialized with BEGIN_ADDRESS again when write commands

are completed to start the read commands.

- This is the address going to the MC

if(rst)

cmd_addr <=  BEGIN_ADDRESS;

else if (cmd_en & app_rdy)

if (cmd_addr < ((NUM_WRITES-1)*8))

cmd_addr <=  cmd_addr + 4'b1000;

// The cmd_cnt value is 99 when it completes 100 write transactions.

// The cmd_addr is initialized with BEGIN_ADDRESS when write commands

// are completed (cmd_cnt = 99) to start the read commands.

else if (cmd_cnt == ((NUM_WRITES-1)))

cmd_addr <= #TCQ BEGIN_ADDRESS;

Write enable signal generation to the MC

- The app_wdf_wren signal is the write enable issued to the MC.

- When the app_wdf_wren is high, the write data is written into the write data fifo.

- The app_wdf_wren signal is asserted for the write command when both MC write fifo (app_wdf_rdy) and MC command fifo (app_rdy) are ready.